APG 8

Zadání úkolu

Zadaná data otevřete v ArcGIS. Zjistíte parametry vrstev a stejná data převedete do GRASS.

Jak nastvit pracovní prostředí GRASSu – postup je níže. Podle nastavení rozsahu vrstev (layers) v ArcGIS (zjistíte si souřadnice S, J, V, Z – pro rozsah území a stejně je zadáte v GRASSu)

Otevřete GRASS a použijete nástroje pro definování nové location: Define new location with projection values

nadefinujete paramtetry s-jtsk a křovákova zobrazení (přesný postup lze najít na netu, ale zde jej máte):

Textové rozhraní – tam se dostanete pro nadefinování nové locatio. Nejprve bude nutné zadat název location (melechov), název mapsetu (cv8), název database = cesta k souborům. Esc+Enter a pokračujeme:

Would you like to create location <t1> ? (y/n) [y] <ENTER>

To create a new LOCATION, you will need the following information:

1. The coordinate system for the database

        x,y (for imagery and other unreferenced data)

        Latitude-Longitude

        UTM

        Other Projection

2. The zone for the UTM database

   and all the necessary parameters for projections other than

   Latitude-Longitude, x,y, and UTM

3. The coordinates of the area to become the default region

   and the grid resolution of this region

4. A short, one-line description or title for the location

 Do you have all this information? (y/n) [y] <ENTER>

Please specify the coordinate system for location <t1>

A   x,y

B   Latitude-Longitude

C   UTM

D   Other Projection

RETURN to cancel

D <ENTER>

Other Projection coordinate system? (y/n) [y] <ENTER>

Please enter a one line description for location <t1>

> popis grass lokace

=====================================================

popis grass lokace

=====================================================

ok? (y/n) [y] <ENTER>

Please specify projection name

Enter 'list' for the list of available projections

Hit RETURN to cancel request

> krovak <ENTER>

Do you wish to specify a geodetic datum for this location?(y/n) [y] <ENTER>

Please specify datum name

Enter 'list' for the list of available datums

or 'custom' if you wish to enter custom parameters

Hit RETURN to cancel request

>hermannskogel <ENTER>

Now select Datum Transformation Parameters

Please think carefully about the area covered by your data

and the accuracy you require before making your selection.

Enter 'list' to see the list of available Parameter sets

Enter the corresponding number, or <RETURN> to cancel request

>2 (3 pro SR)

Plural form of units: meters <ENTER>

<ESC><ENTER>

Do you accept this region? (y/n) [y] > <ENTER>

Hit RETURN <ENTER>

<ESC><ENTER>

GRASS> g.proj -p

-PROJ_INFO-------------------------------------------------

name       : Krovak

datum      : hermannskogel

towgs84    : 570.8,85.7,462.8,4.998,1.587,5.261,3.56

proj       : krovak

ellps      : bessel

a          : 6377397.1550000003

es         : 0.0066743722

f          : 299.1528128000

-PROJ_UNITS------------------------------------------------

unit       : meter

units      : meters

meters     : 1.0

Po tomto nastavení byste měli otevřít GRASS již v nové lokaci. 

Import shp do GRASSu. 

File/Import vector map/Multiple formats using OGR.

Postupně vyberete shp soubory a naimportujete do GRASSu (pro začátek naimportujte jenom vrstevnice a vodní toky).

Vytvoření povrchu z vrstevnic

Převod vrstevnic na rastrové, jako hodnota rastru bude použita výška vrstevnice uvedená ve sloupci HODNOTA.

Interpolace povrchu na základě rastrových vrstevnic:

Raster/Interpolate surfaces/raster contours

Vizualizujte si povrch v Map display a následně ve 3D v NVIZ (třetí ikona zleva v map dispay) a zkuste nastavit vhodný pohled a převýšení.

Pokud zvládnete, vizualizujte povrch společně s vodními toky.

